# Smart Manufacturing Data Hub (SMDH)

# System Requirements Document

# Document Information

| Version | 0.2 (Architectural Review Update) |
| --- | --- |
| Date | 2 November 2025 |
| Status | Updated Based on Architectural Review Feedback |
| Owner | AI Applied |
| Classification | Internal Use |

# 1. Executive Summary

The Smart Manufacturing Data Hub (SMDH) is a cloud-based platform that gives small and medium-sized manufacturing companies real-time visibility of their operations. Companies can register themselves, add their sites and devices, and immediately start collecting and viewing their data through dashboards.

Let manufacturing companies manage their own data from start to finish—device setup to insights—without needing technical knowledge or external help.

# 2. Guiding Principles

## Principle 1: Self-Service Company Onboarding

Companies must be able to register and set up their manufacturing infrastructure themselves through a simple portal.

Companies can sign up without talking to sales

Site administrators can register multiple sites

Users can set up devices and sensors without technical knowledge

Companies can assign and manage their own user roles

The setup process is guided and simple

Key Requirements:

Self-service registration workflow with company verification

Multi-site management capability within a single company account

Role-based user management (administrators, operators, viewers)

Device provisioning wizard with automatic credential generation

Configuration templates for common device types and use cases

Step-by-step guided setup with validation at each stage

## Principle 2: Automated Data Collection and Visualisation

Once registered, the system must automatically receive, store, and display data from configured devices without manual work.

Data flows automatically from registered devices to the platform

Historical data is stored securely

Dashboards are created automatically based on device types

Companies can view their data as soon as devices connect

The system handles all data processing and quality checks

Dashboards update automatically

Key Requirements:

Automated data ingestion pipelines supporting multiple protocols (MQTT, HTTP, LoRaWAN)

Automatic data storage with multi-tenant isolation

Pre-configured dashboard templates by use case (machine utilisation, air quality, job tracking)

Real-time and historical data visualisation

Automated alerts and notifications based on configurable thresholds

Self-service dashboard customisation and report generation

# 3. Business Requirements

## 3.1 Platform Targets

| What | Target | Priority |
| --- | --- | --- |
| Support SME manufacturing companies | 30-40 companies (Year 1), growing to 100 | High |
| Users logged in at once | 20-40 users | High |
| Daily data processing | 2.6M-3.9M rows/day per company | High |
| Uptime | 99.9% (≤43 minutes downtime/month) | Critical |
| Dashboard data refresh for analytics | Within 5 minutes | High |
| Safety/critical alerts | Within 5-10 seconds | Medium |
| Operational alerts | Within 60 seconds | High |
| Operator dashboard refresh | Within 60 seconds for most uses | High |
| Dashboard creation | Within 5 minutes after device registration | High |
| Basic setup completion | Within 30 minutes | Medium |

## 3.2 Target Users

Who will use this:

**Company Administrators** - Register company, manage sites, handle billing

**Site Administrators** - Register devices, manage site users, set up dashboards

**Operators** - View dashboards, get alerts, create reports

**Viewers** - Read-only access to dashboards and reports

What users are like:

Not technical experts in IoT or cloud

Know their manufacturing operations

Need access from mobile and desktop

Need simple, clear interfaces

Want speed and simplicity over advanced options

## 3.3 Business Constraints

Cost: Infrastructure costs must match actual usage and grow linearly with number of companies

Location: Initial deployment in UK/EU (data stored in London region)

Compliance: Must meet GDPR, ISO 27001, SOC 2 Type II standards

Growth: Must support 100+ companies within 3 years

Launch: Phase 1 delivery date to be set after architecture selection

# 4. Functional Requirements

### FR-1: Company Registration

| Requirement ID | Description |
| --- | --- |
| REQ-1.1 | New companies can create accounts with their details (name, address, industry, contact) |
| REQ-1.2 | Email addresses are verified automatically |
| REQ-1.3 | Passwords must be secure (12+ characters, mixed case, numbers, symbols) |
| REQ-1.4 | Multi-factor authentication available for administrators |
| REQ-1.5 | Terms of service and privacy policy must be accepted |
| REQ-1.6 | Each company gets a unique identifier to keep their data separate |
| REQ-1.7 | Creates a default administrator account |
| REQ-1.8 | Sends a welcome email with setup instructions |

Must achieve:

Registration completed in under 10 minutes

All fields checked immediately as typed

Email verification within 5 minutes

Company active immediately after email confirmation

### FR-2: Manufacturing Site Registration

| Requirement ID | Description |
| --- | --- |
| REQ-2.1 | Unlimited sites per company |
| REQ-2.2 | Capture site details (name, address, timezone, operating hours) |
| REQ-2.3 | Site-specific settings |
| REQ-2.4 | Site hierarchy (regions, facilities, departments) |
| REQ-2.5 | Archive/deactivate sites without losing data |
| REQ-2.6 | Site-level reporting |
| REQ-2.7 | Geographic visualisation of sites |

Must achieve:

Sites registered in under 5 minutes

Site available for device registration immediately

Configuration changes take effect within 1 minute

### FR-3: User and Role Management

| Requirement ID | Description |
| --- | --- |
| REQ-3.1 | User roles: Company Admin, Site Admin, Operator, Viewer |
| REQ-3.2 | Company Admins can invite users by email |
| REQ-3.3 | Role-based access control for all resources |
| REQ-3.4 | Users can be assigned to specific sites |
| REQ-3.5 | User activity logs |
| REQ-3.6 | Users can update their own profiles |
| REQ-3.7 | Single Sign-On (SSO) via SAML 2.0 |
| REQ-3.8 | Session timeout and limits on simultaneous logins |

Must achieve:

Invitation emails sent within 1 minute

Role changes take effect immediately

Blocked access attempts are logged

### FR-4: IoT Device Registration

| Requirement ID | Description |
| --- | --- |
| REQ-4.1 | Support registration of various device types: - Machine utilisation sensors (power monitors, state sensors) - Air quality sensors (CO2, VOC, particulate matter, temperature/humidity) - Energy metres (smart metres, power monitors) - RFID readers and barcode scanners - Custom/generic IoT devices |
| REQ-4.2 | Auto-generate device credentials (X.509 certificates, API keys) |
| REQ-4.3 | Provide downloadable configuration files for devices |
| REQ-4.4 | Display connection instructions specific to device type |
| REQ-4.5 | Support bulk device import via CSV/Excel |
| REQ-4.6 | Provide device connectivity testing and validation |
| REQ-4.7 | Allow custom metadata and tags per device |
| REQ-4.8 | Support device grouping and hierarchical organisation |
| REQ-4.9 | Enable device firmware version tracking |
| REQ-4.10 | Provide device health monitoring and connectivity status |

Must achieve:

Device registration in under 3 minutes per device

Credentials work immediately

Devices connect within 1 minute of registration

Connection status updates in real-time

### FR-5: Device Configuration and Provisioning

| Requirement ID | Description |
| --- | --- |
| REQ-5.1 | Provide pre-configured templates for common device types |
| REQ-5.2 | Support custom data schemas via JSON/YAML |
| REQ-5.3 | Auto-detect device schema from initial data transmission |
| REQ-5.4 | Allow configuration of data transmission frequency |
| REQ-5.5 | Support over-the-air (OTA) configuration updates |
| REQ-5.6 | Provide device twin/shadow for state management |
| REQ-5.7 | Enable batch configuration updates across device groups |
| REQ-5.8 | Support A/B testing of configuration changes |

Must achieve:

Templates for 10+ common device types

Schema detection accuracy above 95%

Configuration changes take effect within 1 minute

### FR-6: Automated Data Collection

| Requirement ID | Description |
| --- | --- |
| REQ-6.1 | Support multiple ingestion protocols: - MQTT v3.1.1 and v5 (primary for IoT sensors) - HTTP REST API (for RFID/barcode scanners) - LoRaWAN (via gateway integration) - File upload (CSV, Excel, JSON) |
| REQ-6.2 | Handle data rates up to 1Hz for high-frequency sensors |
| REQ-6.3 | Process batch data uploads up to 1GB per file |
| REQ-6.4 | Provide automatic retry for failed transmissions |
| REQ-6.5 | Support data buffering during network outages |
| REQ-6.6 | Implement dead letter queue for unprocessable messages |
| REQ-6.7 | Validate data against device schema automatically |
| REQ-6.8 | Enrich data with metadata (device ID, timestamp, tenant ID) |
| REQ-6.9 | Support data compression to reduce transmission costs |
| REQ-6.10 | Provide ingestion metrics and monitoring - Daily ingestion: 2.6M - 3.9M data points - Peak throughput: 60 messages/second per device - Batch file processing: <5 minutes for 1GB file |

Must achieve:

Data appears in system within 5 seconds of sending

No data loss for successfully sent messages

Failed messages retry automatically up to 3 times

### FR-7: Data Storage and Retention

| Requirement ID | Description |
| --- | --- |
| REQ-7.1 | Maintain complete data isolation between tenants |
| REQ-7.2 | Store raw, unprocessed data for audit purposes |
| REQ-7.3 | Create normalised, query-optimised data views |
| REQ-7.4 | Implement automatic data aggregation (hourly, daily, weekly) |
| REQ-7.5 | Support configurable retention policies: - Raw data: 90 days - Aggregated data: 2 years - Reports and exports: 5 years |
| REQ-7.6 | Provide Time Travel capability (point-in-time recovery) |
| REQ-7.7 | Enable tenant-specific data export (CSV, JSON, Parquet) |
| REQ-7.8 | Support data anonymisation for tenant offboarding |
| REQ-7.9 | Implement automated backup and disaster recovery |
| REQ-7.10 | Provide storage metrics and usage reporting per tenant - Year 1: 110-150 GB per company - Growth rate: 10-15 GB per month per company - Maximum single object size: 5 GB |

Must achieve:

Data can be queried within 30 seconds of arrival

Company data is completely isolated (verified by audit)

Export requests complete within 15 minutes for 1 year of data

### FR-8: Data Quality and Validation

| Requirement ID | Description |
| --- | --- |
| REQ-8.1 | Validate data types and formats against schema |
| REQ-8.2 | Detect and flag outliers using statistical methods |
| REQ-8.3 | Identify duplicate or missing data points |
| REQ-8.4 | Calculate data quality scores per device |
| REQ-8.5 | Alert administrators when quality drops below threshold |
| REQ-8.6 | Provide data profiling and quality reports |
| REQ-8.7 | Support custom validation rules per device type |
| REQ-8.8 | Automatically handle timezone conversions |
| REQ-8.9 | Detect and correct clock drift in device timestamps |

Must achieve:

Data quality checks complete within 1 second per record

Quality issues flagged in real-time

Quality reports available daily

### FR-9: Automated Dashboard Creation

| Requirement ID | Description |
| --- | --- |
| REQ-9.1 | Auto-provision default dashboards based on device type: - Machine Utilisation: OEE, energy consumption, uptime/downtime - Air Quality: Real-time readings, compliance status, trend analysis - Job Tracking: Production flow, cycle times, bottleneck analysis - Energy Management: Consumption patterns, cost analysis, efficiency |
| REQ-9.2 | Make dashboards available within 5 minutes of first data point |
| REQ-9.3 | Support real-time updates (1-second refresh for critical metrics) |
| REQ-9.4 | Provide historical trending and comparison views |
| REQ-9.5 | Enable drill-down from summary to detailed views |
| REQ-9.6 | Support custom date range selection |
| REQ-9.7 | Provide export functionality (PDF, PNG, CSV) |
| REQ-9.8 | Enable dashboard sharing via secure links |
| REQ-9.9 | Support mobile-responsive design |

Must achieve:

Dashboards load in under 2 seconds

Real-time data updates within 5 seconds

Dashboards work on iOS and Android

Exports complete in under 30 seconds

### FR-10: Self-Service Reporting

| Requirement ID | Description |
| --- | --- |
| REQ-10.1 | Provide report builder with drag-and-drop interface |
| REQ-10.2 | Support scheduled report generation (daily, weekly, monthly) |
| REQ-10.3 | Enable email delivery of reports |
| REQ-10.4 | Provide report templates for common analyses |
| REQ-10.5 | Support multi-site and multi-device aggregation |
| REQ-10.6 | Allow custom KPI definition and tracking |
| REQ-10.7 | Enable report sharing within company users |
| REQ-10.8 | Support multiple output formats (PDF, Excel, CSV, HTML) |
| REQ-10.9 | Provide report execution history and audit trail |

Must achieve:

Non-technical users can create reports in under 10 minutes

Scheduled reports arrive within 15 minutes of scheduled time

Reports can combine data from up to 100 devices

### FR-11: Automated Alert System

| Requirement ID | Description |
| --- | --- |
| REQ-11.1 | Support threshold-based alerts (above/below value) |
| REQ-11.2 | Enable anomaly detection using machine learning |
| REQ-11.3 | Provide multi-channel notifications: - Email - SMS (for critical alerts) - In-app notifications - Push notifications (mobile app) - Webhook integrations |
| REQ-11.4 | Support alert priority levels (low, medium, high, critical) |
| REQ-11.5 | Enable alert acknowledgement and resolution workflow |
| REQ-11.6 | Provide alert escalation rules |
| REQ-11.7 | Support alert suppression during maintenance windows |
| REQ-11.8 | Allow custom alert templates per device type |
| REQ-11.9 | Provide alert analytics and reporting |
| REQ-11.10 | Support alert rule testing before activation - CO2: Warning at >1000 ppm, Critical at >5000 ppm - PM2.5: Warning at >35 μg/m³, Critical at >150 μg/m³ - VOC: Warning at >220 ppb, Critical at >660 ppb - Temperature: Warning at <16°C or >30°C - Machine offline for >15 minutes - Energy consumption >20% above baseline - OEE drops below 70% for 1 hour - Predictive maintenance required |

Must achieve:

Alerts trigger within appropriate time based on criticality (see note below)

Email notifications sent within 1 minute

SMS notifications sent within 30 seconds for critical alerts

Alert acknowledgement shown in system within 1 second